# Montage tomography Serial-EM SOP

- Make folder for acquisition, this will store atlases, gridsquares and the montages theselves when SerialEM prompts you on start up.

- Make Atlas (remember to make Navigator map) and align to lowest SA mag (

- Setup low dose presets:

  - View, nanoprobe 6400 x mag (lowest SA mag where  beam covers camera)

  - Record, get beam as close as possible to size of camera

  - Focus, same mag and spot size as exposure but make sure beam covers camera

- Perform microscope alignments

  - Move to vacuum

  - C2 aperture alignment and astigmatism

  - Beam tilt pivot points

  - Filter alignments (tune gif)

  - Gain reference (in GMS, also acquire record image with beam size to be used in acquisition)

  - Dose (measure e/pix/s in GMS, calculate e/A/s and work out exposure time)

  - Move to carbon

  - Rotation centre (this is really important!)

  - Objective astigmatism

  - Objective coma

  - Beam tilt pivot points

  - Beam shift

  - View – Record alignment in Serial-EM

  - Use Determine overlap_function.py script to work out your beam overlap. Write these values into the Setup_square_montage.py script. Run the scripts otherwise these values wont be updated!

- Acquire search montages:

  - Add points to Atlas in middle of Grid squares and tick “acquire” for each of these points in the navigator

  - File -> Open New, call the new file “searchmontages.mrc”

  - File -> set up Montage, make sure that stage

  - Navigator -> Acquire at items, remember to tick “make navigator maps”

- Add montage positions (or polygons) to searchmontages maps.

  - Select to acquire each of these in the navigator.

  - File -> Open New, call the new file “ViewImages.mrc”

  - Use “Acquire at items” and make sure that the following settings are chosen:

    - images are being acquired at view preset

    - New navigator maps are being made at each point

    - perform rough and fine eucentric jobs before acquisition

    - Run “Setup_square_montage.py” or “Setup_square_montage_from_polygon.py” after acquisition as appropriate

  - Check that the label in “xxx” “Montage_imageshifts_xxx.txt” files corresponds to the navigator labels of the view images.

- Before running the acquisition script check:

  - Rotation centre

  - Tilt pivot points

  - Beam centering

  - Focus area in low dose (can edit focus area per montage using “Edit Focus” box in the navigator)

  - Acquisition time is what you intended it to be.

- Select the new view image maps in the navigator for acquisition and choose acquire at items choose:

  - To run the “Montagetiltseries” script at each item.

  - Close column valves at end.